From dbraun@iil.intel.com Mon Nov 15 05:58:39 1993

From: dbraun@iil.intel.com (Doug Braun)

Date: Mon, 15 Nov 1993 15:41:47 +0200

To: nec-list@ee.ubc.ca

Subject: Nifty utility to use with NEC

Cc: dbraun@iil.intel.com

Content-Length: 4024

Status: RO

X-Lines: 120

One problem with NEC is that there are few ways to parameterize

any of the input data points. If you have a complex geometry and

you want to change it, you may have to re-calculate and edit a

whole binch of numbers. The geometry rotation, translation, and

reflection commands are useful, but not enough.

I wrote a little perl utility, "param". Param searchs for and

evaluates algebraic expressions in the input file, and replaces

them with their actual values. Below is the script, a sample

input file, and the results of running it through the script.

(Note that in this example, I could have used the GM command to

do essentially the same thing, but I didn't have the

documentation for the GM cpmmand at the time. By the way, I

hacked nec.f a bit sdo it wouldn't choke on blank lines, and

extra text after all the parameters on a line. This is not

required for "param" to work, though.

Sample input file:

****************************************************************************************************8

# $diplen = 0.430     # Dipole length (approx 1/2 wave)

# $sep = 0.30	  # separation (diameter) of opposite dipoles   50

# $angle = 30        # tilt angle    48

# $wire = .005

# $theta = 2*3.1416/360*$angle

CE  299.7-mhz CP Lindenblad Antenna

GW  1  7  {-$sep/2} {-$diplen/2*cos($theta)} {-$diplen/2*sin($theta)}   {-$sep/2}  {$diplen/2*cos($theta)}  {$diplen/2*sin($theta)}  {$wire}	 Driven element

GW  2  7  {-$diplen/2*cos($theta)} {$sep/2} {-$diplen/2*sin($theta)}   {$diplen/2*cos($theta)} {$sep/2} {$diplen/2*sin($theta)}  {$wire}	 Driven element 2

GW  3  7  {$sep/2} {$diplen/2*cos($theta)} {-$diplen/2*sin($theta)}   {$sep/2}  {-$diplen/2*cos($theta)}  {$diplen/2*sin($theta)}  {$wire}	 Driven element 3

GW  4  7  {$diplen/2*cos($theta)} {-$sep/2} {-$diplen/2*sin($theta)}   {-$diplen/2*cos($theta)} {-$sep/2} {$diplen/2*sin($theta)}  {$wire}	 Driven element 4

GE

PT  -1

EX  0  1  3  0  1.0

EX  0  2  3  0  1.0

EX  0  3  3  0  1.0

EX  0  4  3  0  1.0

RP  0 9  3 1110   90.0   0.0  -10.0  22.5

XQ

EN

****************************************************************************************************8

Output of "param", to be fed to NEC2:

****************************************************************************************************8

CE  299.7-mhz CP Lindenblad Antenna

GW  1  7  -0.15 -0.186195 -0.1075   -0.15  0.186195  0.1075  0.005	 Driven element

GW  2  7  -0.186195 0.15 -0.1075   0.186195 0.15 0.1075  0.005	 Driven element 2

GW  3  7  0.15 0.186195 -0.1075   0.15  -0.186195  0.1075  0.005	 Driven element 3

GW  4  7  0.186195 -0.15 -0.1075   -0.186195 -0.15 0.1075  0.005	 Driven element 4

GE

PT  -1

EX  0  1  3  0  1.0

EX  0  2  3  0  1.0

EX  0  3  3  0  1.0

EX  0  4  3  0  1.0

RP  0 9  3 1110   90.0   0.0  -10.0  22.5

XQ

EN